Architect

Facility Name
Project Number

Project Name



Project Location

[image: image1.jpg]DIAMOND KOTE

Building Products




7102 Commerce Drive
Schofield, Wisconsin 54476
www.diamondkotesiding.com
Phone: 800-236-1528
Diamond Kote® Building Products is an industry leading brand of pre-finished engineered wood siding products, manufactured by Wausau Siding Systems™. The innovative siding system provides solutions that deliver value for builders, architects and homeowners. Diamond Kote® Pre-finish is an advanced paint technology providing lasting durability backed by a 30-Year No Fade Warranty. Diamond Kote® is a certified LP® SmartSide® Preferred Pre-finisher and has been an innovative leader in technology and pre-finishing. For more information about Diamond Kote®, visit www.diamondkotesiding.com.
INSTRUCTIONS TO SPECIFIER

This guide specification is intended to be edited for specific project requirements. However, Diamond Kote® and Wausau Supply® are not responsible for edited versions of this section used for individual projects.

This guide specification section has automatic paragraph numbering to facilitate editing. The styles are compatible with styles used in Avitru MasterSpec® so that the edited section can be easily reformatted to the user’s preferences.

Coordinate cross references to other specification sections in this section with actual specification section numbers and titles.

Text in bold within square brackets [ ] are editing options. Angle brackets < > are used to indicate where user text is to be inserted.

Composition siding is often priced against fiber cement siding which is specified in Section 074646.
SECTION 068200 - PRE-FINISHED COMPOSITION WOOD TRIM
Be sure to display hidden notes to specifier. (Click Here to learn how.)
PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:
1. Factory-finished engineered wood post wraps.

2. Factory-finished engineered wood trim.

1.2 ADMINISTRATIVE REQUIREMENTS
A. Coordinate installation of wood trim and adjoining construction.
1.3 PREINSTALLATION MEETINGS

Use this article if the project is large or complex.

A. Meet at Project site <insert number> days before starting installation.

Diamond Kote may have the ability to send a technical representative to a project site based on the location of the project.  Contact Diamond Kote® at 800-236-1528 for information.

B. Attendees: Contractor, Installer, Diamond Kote® Technical Representative, and affected trades.
C. Discussion Topics: 

1. <Insert specific requirements>

1.4 ACTION SUBMITTALS

A. Product Data:

1. Engineered wood post wraps.

2. Engineered wood trim.

3. Sealant.

4. Accessories.

B. Sustainable Design Submittals:

Supbaragraphs below pertain to LEED 2009 and LEED v4.  LP® SmartSide® products may be certified by the Sustainable Forestry Initiative (SFI).  SFI was named an Alternative Compliance Path provider to FSC-certified wood.

Diamond Kote® has production facilities in the following locations:

· 7102 Commerce Drive Schofield, WI 54746

· 803 SW 8th Street Stuart, IA 50250

· 3356 Dyess Avenue Rapid City, SD 57701

1. Certified Wood: Certificates from FSC-alternative compliance body. Confirm wood products comply with forest certification requirements.  Include statement indicating cost for each certified wood product.

2. Recycled Content: Include statement indicating costs and percentage content of preconsumer and postconsumer recycled materials.
3. Location of Manufacture: Indicate distance to Project and fraction by weight of each regionally manufactured and extracted material.

C. Shop Drawings:

1. Include details of construction and installation.

Retain "Samples for Initial Selection" if Architect has not preselected a texture/color or if a custom color will be specified.

D. Selection Samples: Submit manufacturer's full range of standard colors and textures.
Retain "Samples for Verification" Paragraph below to request samples of any or all specified products.

E. Verification Samples: Submit each exposed product, color, and texture specified, in sizes as follows:
Manufacturer's standard linear trim samples are 9 inches long by full width.  Larger samples are available upon request.

1. Linear Trim: 9 inch long by full width.

2. Other Trim: Full size unit. 

3. Prefinished Accessories: Full size.

1.5 INFORMATIONAL SUBMITTALS

A. Manufacturer Certificates:  Signed by manufacturer certifying engineered wood complies with requirements specified.

B. Product Test Reports: Based on evaluation of comprehensive tests performed by manufacturer and witnessed by a qualified testing agency.
1. ICC-ES ESR-1301.

2. APA PR-N124.
1.6 CLOSEOUT SUBMITTALS

A. Manufacturer's Care and Maintenance Guide.

1.7 QUALITY ASSURANCE

Review Division 01 requirements for installer qualifications to verify this language is not in conflict.

A. Installer Qualifications: Firm or individual experienced in installing prefinished wood siding, with a record of successful performance.
Mockup is suggested, particularly for multi-story projects.
B. Mockup: Build engineered wood [column wrap] [trim] mockup, including accessories, to establish quality material and installation standards.
1. Typical column wrap.
2. Acceptable mock-ups may remain as part of the Work if undamaged at time of Substantial Completion.

C. Sustainable Design Qualifications:

1. Manufacturer and Fabricator: Certified by an FSC-accredited alternative compliance path certification body.

1.8 DELIVERY, STORAGE, AND HANDLING

A. Deliver, store and handle materials and products in strict compliance with manufacturer's instructions and industry standards.

B. Store products in manufacturer's labeled packaging until ready for installation. Protect from damage.

C. Store products off ground, level, and under waterproof covering or roof.

1.9 FIELD CONDITIONS

A. Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer. Do not install products under environmental conditions outside manufacturer's limits.

1.10 WARRANTY

Refer to https://www.diamondkotesiding.com/warranty for information about Diamond Kote's warranties.

A. Manufacturer's Standard Warranty: Transferable limited warranty.
1. Labor and Replacement Warranty: Manufacturer agrees to provide replacement materials and reimburse labor to repair or replace products that fail in materials or workmanship within five years from date of Substantial completion.

2. Material Warranty: Manufacturer agrees to provide replacement materials to repair or replace products that fail in materials or workmanship from years six through fifty after date of delivery of the material.

3. Failures include the following:

a. Structural failures including buckling.

b. Deterioration of materials beyond normal weathering.

c. Fungal degradation.
d. Cracking, peeling, separating, chipping, flaking, or rupturing of resin-impregnated surface overlay.

e. Hail damage consisting of a crack, chip, or dent in the surface overlay exceeding 3/8 inch (9.5 mm) in length or diameter.  Warranty covers hail up to 1-3/4 inches (44.5 mm) in diameter.
B. Finish Warranty: Manufacturer agrees to repair finishes that deteriorate within specified warranty period.

1. Deterioration includes the following:

a. Discoloring due to chalking more than a No. 8 rating when tested according to ASTM D4214.
b. Peeling or blistering.

c. Eroding and exposing the substrate.

d. Discoloring due to yellowing.

e. Discoloring due to fading more than 3 Delta E when tested according to ASTM D2244.
2. Warranty Period: 

a. Thirty years, with prorated application costs after year seven and material-only after year fifteen.  
PART 2 -  PRODUCTS

2.1 APPROVED MANUFACTURERS

A. Manufacturer: Subject to compliance with requirements, provide Diamond Kote® Building Products built on LP® SmartSide®. 7102 Commerce Drive, Schofield, Wisconsin 54476; https://diamondkotesiding.com/; (800) 236-1528.
B. Source Limitations: Obtain products, including related accessories, from single source from single manufacturer.
C. No Substitutions.
2.2 MANUFACTURED PRODUCTS

A. Prefinished Strand Siding Materials:

1. Description: Exterior-grade phenolic resin-saturated paper overlay laminated to EPA-registered zinc-borate-preservative-treated engineered wood siding; structurally rated; AWPA compliant; exposed edges sealed for moisture resistance; factory finished.
Include the following paragraph for sustainable products. 
B. Certified Wood: Wood products certified through qualified alternative compliance paths.
2.3 POST WRAPS
See https://www.diamondkotesiding.com/exterior-accents/#propostwrap for more information
A. Strand Panel Post Wraps: Pro-Post Wrap™

1. Description: Pre-finished strand siding material fabricated with interlocking joint.

2. Thickness: 0.53 inches (13 mm).

3. Width: 

a. 4 by 4 (89 x 89 mm) Posts: Four 5 inch (127 mm) boards per post.

b. 4 by 6 (89 x 140 mm) Posts: Two 5 inch (127 mm) boards and two 7 inch (178 mm) boards per post.

c. 6 by 6 (140 x 140 mm) Posts: Four 7 inch (178 mm) boards per post.

4. Board Length: 12 feet (3658 mm).

5. Texture: Woodgrained. 
6. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

7. Post Wrap Accessories:

a. Glue Kit: Manufacturer's standard.

2.4 TRIM

Includes fasciae, arches, rosettes, nail fins, RigidMount items, etc

A. Arch Trim: Pre-finished strand siding material arch trim.

1. Thickness: 0.91 inches (23 mm).

2. Width: [3-1/2 inches (89 mm)] [5-1/2 inches (140 mm)]

3. Size and Radius: See Drawings for custom sizes.

4. Texture: Woodgrained.

5. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

Keystones offered in two sizes: 4 inch and 6 inch.

B. Keystones: Pre-finished strand siding material keystone.

1. Thickness: 0.91 inches (23 mm).

2. Size: [5-1/2 inches (140 mm) wide by 4-1/2 inches (114 mm) high] [7-1/2 inches (191 mm) wide by 6-1/2 inches (165 mm) high].

3. Texture: Woodgrained.

4. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.
Nail Fin Trim and Nail Fin Corners are applied to the structure via the plastic nailing fin, eliminating the need for face nails and associated touch up painting.

C. Rabbeted Nail Fin Trim: Pre-finished strand siding material manufactured with a J-Pocket and integral plastic nailing fin. 

1. Thickness: 0.91 inches (23 mm).

2. Exposure: [3-1/2 inches (89 mm)] [5-1/2 inches (140 mm)] 

3. Length: 16 feet (4877 mm).

4. Texture: [Woodgrained] 
5. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

D. Inside Corner Nail Fin Trim: Pre-finished strand siding material manufactured with integral plastic nailing fin. 

1. Thickness: 0.91 inches (23 mm).

2. Exposure: [1-3/4 inches (44 mm)] each leg.

3. Length: 10 feet (3048 mm).

4. Texture: Woodgrained.

5. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.
E. Rabbeted Nail Fin Outside Corner : Pre-finished strand siding material manufactured with J-Pocket and integral plastic nailing fin.

1. Thickness: 0.91 inches (23 mm).

2. Exposure: [3-1/2 inches (89 mm)]  each leg.

3. Length: [10 feet (3048 mm)] [16 feet (4877 mm)].

4. Texture: [Woodgrained] 
5. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.
F. Outside Corner Nail Fin Trim: Pre-finished strand siding material manufactured with integral plastic nailing fin. 

1. Thickness: 0.91 inches (23 mm).

2. Exposure: [3-1/2 inches (89 mm)] [5-1/2 inches (140 mm)] each leg.

3. Length: [10 feet (3048 mm)] [16 feet (4877 mm)].

4. Texture: [Woodgrained] [Smooth]
5. Color: [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.
G. Strand Trim: Solid prefinished trim members; same material as cladding.

1. Thickness: [0.53 inch (13.5 mm)] [0.625 inch (15 mm)] [0.91 inch (23 mm)].

2. Width: [1-1/2 inch (38 mm)] [2-1/5 inch (63 mm)] [3-1/2 inch (89 mm)] [4-1/2 inch (114 mm)] [5-1/2 inch (140 mm)] [7.2 inch (184 mm)] [9.2 inch (235 mm)] [11.2 inch (286 mm)].

3. Length: 16 feet (4877 mm).

4. Texture: [Woodgrained] [Smooth]
5. Color: [Match cladding] [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.
RigidMount are premanufactured blocks of strand siding material accommodating electrical outlets, hose bibs, dryer vents, cable penetrations, and house numbers.  Refer to https://www.diamondkotesiding.com/light-and-utility-blocks/ for information.
H. Siding Accessories: Pre-finished strand siding material manufactured with built-in drip cap and concealed nailing flanges.
1. Products:

a. RigidMount™ UL Electrical: Strand siding material trim with built-in junction box, 5.94 by 6.94 inches (151 mm by 176 mm).

b. RigidMount™ Receptacle: Strand siding material trim without built-in junction box, 5.88 by 7.69 inches (149 mm by 195 mm).

c. RigidMount™ Blank Universal: Strand siding material trim, 8.25 by 10.313 inches (210 mm by 262 mm).

d. RigidMount™ Universal: Strand siding material trim with pass through for utility, 8.25 by 10.313 inches (210 mm by 262 mm).
e. RigidMount™ Split Block: Two-piece strand siding material trim with adjustable sized pass through for utility, 5.88 by 7.69 inches (149 mm by 195 mm).

f. RigidMount™ Oversize: Strand siding material trim, 8.25 by 14.313 inches (210 mm by 364 mm).

g. RigidMount™ Blank Horizontal: Strand siding material trim, 15.875 by 8.75 inches (403 mm by 222 mm).

h. Personalized RigidMount™: Strand siding material trim, 15.875 by 8.75 inches (403 mm by 222 mm) with customized alphanumeric characters routed in [black] [white] finish. <Insert Characters>.
2. Texture: [Woodgrained] [Smooth]
3. Color: [Match adjacent siding] [See Drawings] [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

2.5 ACCESSORIES

A. Fasteners: ASTM A153, hot-dip galvanized or stainless steel nails; size recommended by manufacturer to achieve proper penetration of substrate.
Color-matched fastener heads are available from Diamond Kote® to eliminate touching up nail heads.

1. Colored Fasteners: Prefinished nails, color to match siding.

B. Sealant: ASTM C920, minimum Class 25 sealant, type recommended by siding manufacturer.

C. Touch-Up Paint: Prefinished composition siding manufacturer's standard touch-up paint provided in 8 oz, quart and gallon bottles in colors matching siding. 

2.6 FINISHES

A. Factory Wood Finishing: 
1. Pre-finished composition siding manufacturer's oven-cured, water-based coating with metal oxide pigments.
2. Basis of Design: Subject to compliance with requirements, provide Diamond Kote® Pre-finish.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Verify concealed framing for support and anchorage of prefinished composition [post wrap] [and] [trim].

B. Verify that substrate has been installed to permit proper installation of pre-finished composition [post wrap] [and] [trim].

3.2 PREPARATION

A. Prepare substrates using manufacturer's recommended methods.

B. Do not install until substrates have been properly prepared and deviations from manufacturer's recommended tolerances are corrected. 

C. Commencement of installation constitutes acceptance of conditions.

3.3 INSTALLATION

A. Install in accordance with manufacturer's instructions.

B. Do not cut cladding to fabricate trim; use trim components.

C. Seal around penetrations. 
D. Paint exposed cut edges, blemishes, and unfinished exposed fasteners with siding manufacturer's touch-up paint.

3.4 ADJUSTING AND CLEANING

A. Remove damaged, improperly installed, or otherwise defective materials and replace with new materials.

B. Clean finished surfaces according to manufacturer's written instructions.

3.5 PROTECTION

A. Protect installed products until Substantial Completion.

B. Touch-up, repair or replace damaged products.

END OF SECTION | 10.18 


074623 - Engineered Wood Siding



Page 1

